Chapter 3.10 Common Network Environments, Connectivity and Security Issues

3.10.a Organising LANs and WANs

Most of this is covered in Chapter 1.6 in the AS text.  Remember that questions may be asked on any part of the A Level Computing Specification in the exam for module 3.

In this Chapter you will learn how to connect LANs and WANs.  Section 3.7.b showed how analogue signals are used from your home PC (or network) to the local telephone exchange.  This connection is analogue if a modem is used.  From then on digital signals are used until the final local exchange.  From this exchange analogue signals must be used if the ordinary home telephone and modem are used by the receiver.

LANs use digital signals to transfer data between nodes.  The rate of transmission of the data depends on the topology of the network and the transmission medium used to join nodes in the network.  Fig. 3.10.a.1 shows a ring network.  The most common medium used in this type of network is unshielded twisted pair (UTP) as described in Section 3.10.b.  This makes ring networks easy to install but limits bandwidth and, therefore, the maximum speed of the network.

Station

Repeater

Fig. 3.10.a.1

Media, other than UTP, are used in ring networks, details of which are given in Table 3.10.a.1.  You are not expected to remember the exact transmission rates and other details.  However, you do need to remember the relative details.

Details of these media are given in Section 3.10.b.

| Medium | Data Rate Mbps | Max. Repeater spacing km | Max. Number of Repeaters |
| --- | --- | --- | --- |
| UTP | 4 or 16 | 0.1 | 72 |
| Shielded TP | 4 or 16 | 0.3 | 260 |
| Baseband Coaxial | 16 | 1.0 | 250 |
| Optical fibre | 100 | 2.0 | 240 |

Table 3.10.a.1

In bus networks the communication network is simply the transmission medium.  Bus networks can use any medium and details are given in Table 3.10.a.2.

| Medium | Data Rate Mbps | Max. Range km | Max. Number of Taps |
| --- | --- | --- | --- |
| UTP | 1, 10 | 2 | 10s |
| Baseband Coaxial | 10 | 2.5 | 100s |
| Broadband coaxial | 10 or 20 per channel | 30 | 1000s |

Table 3.10.a.2

The limits on transfer rates given in the two tables are typical but they are being extended all the time as technology advances.

3.10.b Transmission Media

Unshielded twisted pair (UTP) consists of two conductors, enclosed in a plastic sheath, twisted together.  The pair is contained inside another plastic sheath to keep them together.  There are different types of UTP, giving different data rates as shown in Tables 3.10.a.1 and 3.10.a.2.  Often the outer sheath contains a number of twisted pairs, four being common.  Fig. 3.10.b.1 shows the arrangement of a single twisted pair.  Shielded twisted pair has a grounded screen around the conductors to shield the cable from extraneous electromagnetic radiation.

Fig. 3.10.b.1

The other main type of cable used in LANs is coaxial cable.  This has a central conductor enclosed in a plastic sheath which is surrounded by a copper sheath.  This copper screen is surrounded by a plastic coating as shown in Fig. 3.10.b.2.

Central conductor

Plastic insulators

Copper screen conductor

Fig. 3.10.b.2

The transfer rates for these media are given in the Tables in Section 3.10.a.

Sometimes it is very difficult to lay cables so low-power radio may be used.  This uses radio signals between networks and nodes, with other forms of media used to link other parts of a network together.  This is now being used in schools that have mobile classrooms, sometimes known as demountables.

3.10.c Network Components

Switches use the same type of wiring as hubs (see Section  3.7.d).  However, each connector has full network speed.  A typical layout is shown in Fig. 3.10.c.1.  Here, each station has full speed access to the server.  However, if any of these stations wish to access the main network, they would have to share the connection to the main network.

S

W

I

T

C

H

To main network

Server

Stations

Fig. 3.10.c.1

If the number of stations is increased and they all want to access the main network, the increased local speed would be less useful because of sharing access to the main network.  In a case like this, it may be necessary to upgrade the link to the main network.

A router is used to connect different types of network together.  A router can alter packets of data so that two connected networks (LANs or WANs) need not be the same.  Routers use network addresses and addresses of other routers to create a route between two networks.  This means that routers must keep tables of addresses.  These tables are often copied between routers using routing information protocol (RIP).

Routers enable public networks to act as connections between private networks as shown in Fig. 3.10.c.2.

LAN

Router

Public network

Router

LAN

In order to route data round a network, a router takes the following steps.

- Router receives a network level packet.

- Network destination address is checked.

- Packet is sent to the appropriate network.

Note that, in the case of the Internet, the destination address is the IP address.

Usually a router is slower than a bridge.  A bridge links two LANs which may, or may not, be similar.  It uses packets and the address information in each packet.  To route data efficiently, a bridge learns the layouts of the networks.

Suppose a bridge is used to link two segments together that are not far apart, say in the same building.  The two segments can work independently but, if data needs to go from one segment to another, the bridge will allow this.  Fig. 3.10.c.3 shows this situation.

Segment

Bridge

Segment

Fig. 3.10.c.3

The bridge has to learn where each node is situated.  The bridge will receive data that does not have to be passed from one segment to another.  Initially, any data the bridge receives is buffered and passed to both segments.  The bridge stores a table containing the addresses of sending nodes and the segment from which the data was sent.  Eventually, when all nodes have sent data, the bridge will know on which segment each node is.

Now, when the bridge receives data being sent from one node to another, it can make a decision whether, or not, the receiving node is on the same segment as the sending node.  This leads to the following algorithm.

Sending node sends data onto its segment.

- Data arrives at the bridge and is buffered.

- Bridge checks destination address.

- If destination is on same segment as sender then

  - discard the data

- Else

  - pass data to other segment.

This algorithm will work on the configuration shown in Fig. 3.10.c.3 but now look at Fig. 3.10.c.4 which shows multiple bridge connections.

Bridge A

Bridge B

Bridge C

Fig. 3.10.c.3

Now data can reach bridge B by two different routes from different directions.  This means that the bridge cannot build up its tables properly.  The above algorithm needs modifying and this is done by the spanning tree algorithm (STA) which builds up a picture of multiple connections and blocks one of them.  In Fig. 3.10.c.4, bridge C could be blocked.  In this particular case, any one of the bridges may be blocked.  The actual details of how the STA works is not required at this Level.

Bridges can also be used remotely as shown in Fig. 3.10.c.5.

Bridge

Bridge

Leased line - ISDN

Fig. 3.10.c.5

In this case, the link will affect the overall performance of the LAN.  If there is a great deal of traffic between the segments, and the link is slow, the overall performance will be slow.  In this type of configuration placing a server can be critical.

Bridges are very useful devices because

- they increase the number of stations on a network,

- they enable connections between different types of network,

- the network is treated as a single LAN,

- they improve manageability of the network,

- they improve network security,

- they increase reliability because segments are separate,

- they increase availability because segments are separate.

However, bridges

- introduce delays,

- can be overloaded.

Modems are needed to convert analogue data to digital data and vice versa.  A modem combines the data with a carrier to provide an analogue signal.  This means that ordinary telephone lines can be used to carry data from one computer to another.  This was explained in Section 3.7.b.

3.10.d Common Network Environments

Probably the largest network in use is the Internet.  The internet provides facilities to link computers world-wide, usually using telecommunications systems.  It allows fast communications between people, the transfer of data between computers and the distribution of information.

Messages are passed from the source computer, through other computers, to the destination computer.

The Internet provides

- the World Wide Web,

- electronic mail,

- file transfer.

In order for this system to work, there are Internet Service Providers (ISP) who connect a subscriber to the backbone of the Internet.  These providers then pass data between them and onto their respective clients.  Fig. 3.10.d.1 (on the next page) shows how data, including electronic mail (see Section 3.10.g), are passed from one computer to another.

An intranet is a network offering the same facilities as the Internet but solely within a particular company or organisation.

An intranet has to have very good security for confidential information.  Sometimes the organisation allows the public to access certain parts of its intranet, allowing it to advertise.  This Internet access to an intranet is called an extranet.

Suitable software is required to make these systems work.  Browsers allow a user to locate information using a universal resource locator (URL).  This is the address for data on the Internet.  The URL includes the transfer protocol to be used, for example http, the domain name where the data is stored, and other information such as an individual filename.

e.g. http://www.bcs.org.uk/  will load the British Computer Society's home page.

Domain names are held in an hierarchical structure.  Each name is for a location on the Internet.  Each location has a unique name.  The names in the various levels of the hierarchy are assigned by the bodies that have control over that area.

Consider the domain name

PC195-staff.acadnet.wlv.ac.uk

The domain is uk and the ac would be assigned to a particular authority.  (In this case UKERNA).  This authority would then assign the next part, i.e. wlv.  As this is Wolverhampton University, it is responsible for all the parts prior to wlv.  Those in charge of acadnet are responsible for PC195-staff.

Each computer linked to the Internet has a physical address, a number called its IP (Internet protocol) address.  This numeric address uniquely identifies the physical computer linked to the Internet.  The domain name server converts the domain name into its corresponding IP address.

B

C

D

E

F

G

H

H is destination computer

G is the server for the network H is on

A is source computer

B is the server for the network A is on

C is the internet provider for A

A

A and G are linked via their servers and the nodes C, D, E and F

Fig. 3.10.d.1

3.10.e Hypertext Links

The World Wide Web stores vast amounts of data on machines that are connected to the Internet.  This data may be in the form of text, databases, programs, video, films, audio and so on.  In order to view this data you must use a browser such as Internet Explorer or Netscape.  However, the browser will need to know how to retrieve and display this data.

All the data is situated on computers all over the world.  These computers have unique addresses and the data is held in folders on these computers.  However, not all computers use the same hardware and software.  This means that there must be some protocol that allows all the computers to communicate and be able to pass the data from one computer to another.  One of the protocols to do this is the hypertext transfer protocol (http) that is used by the browsers to receive and transmit data.  A typical URL is

http://www.bcs.org.uk/

which was explained in the previous Section.  Here, the URL starts http:// where http tells the browser which protocol to use.  the portion :// is a separator marking off the transmission protocol from the rest.  This URL connects the user to the home page of the British Computer Society.  If a particular piece of data is required, such as a weather forecast, you can specify a folder to move to directly.  This one

http://bbc.co.uk/weather/

loads a page from the directory weather at bbc.co.uk.  In turn, this page will have links to other directories and pages.

This means that the browser now knows where to look for the data.  Links may be placed so that a user can quickly move around a document or to another document, which may be at a completely different site.  Fig. 3.10.e.1 shows links to documents that are at the same site as the document containing the links.

Smart Cards

Contents

Definitions

Applications

The Electronic Purse

Home page

Fig. 3.10.e.1

The links are usually displayed in a different colour to the rest of the text and are underlined.  When you place your pointer on a link, the pointer becomes a pointing finger.  If you now click the mouse button you will be connected to the appropriate site and the data will be downloaded.  (Try it.).  In this document, when you leave the pointer on a link, the URL will be displayed.  Fig. 3.10.e.2 shows part of the page that is displayed when Applications is clicked on.

Applications

Electronic Purse

Access Control and Security

Travelling

The Future

Smart Card Contents

Home Page

Electronic Purse

This acts like cash. The card can be charged up at modified automatic teller machines (ATMs), modified BT payphones and at new points installed by the provider.  Mondex is one of the largest suppliers of these smart cards and trials are taking place at Aston, Exeter and York Universities as well as at Swindon. The card is loaded with electronic cash and it can then be used to pay for goods and services in a similar way to using a charge card. The difference being that 'cash' is being transferred from the card to the retailer. The cards can transfer 'cash' from one card to another. Thus, if two people, such as a parent and a child, each have a card, the parent can transfer the child's pocket money from one card to the other.

Another large provider of smart cards is Visa. They produce both disposable and reloadable cards. Visa Cash, as it is called, can provide secure trading on the Internet as well as facilities similar to those of Mondex. Click here for more details.

Start of Applications

Access Control and Security

The cards can be used to control access to buildings, rooms and facilities such as libraries. They can hold personal details of the user including biometric data. South Africa is using smart cards to issue pensions to those entitled to them in Kangwane - a former black homeland. Pensioners insert their cards into a machine which is brought to Kangwane once a month, they then pass their finger over a tiny scanner which reads the fingerprint, and checks it with the data on the card. This method of payment has now spread to other provinces and is used to dispense about 400,000 pensions. The scheme is provided by Cash Paymaster Services (CPS), a company set up by First National Bank (FNB) and Dakator. Dakator has now been bought out by FNB who have extended their services to Namibia and the Western Cape.

Further details of links are given in the next Section.

On this page you will see further links, most of which link you to different sites around the world.  For example, Mondex links you to mondex.com, the home page for Mondex who specialise in applications of smart cards.

3.10.f Hypertext Markup Language (HTML)

Using http, your browser can transfer data between computers.  However, the browser still needs to know how to display the data.  This is done by using the hypertext markup language (HTML).  You are not expected to be able to produce detailed HTML in the examination.  However, you may find it useful to remember a few examples in order to explain an answer to a question.

HTML uses tags to indicate how to display the data.  Tags are enclosed in angle brackets - < and >.  For example <B>.  Some tags have two parts.  One indicates the start point and the other the end point.  For example

<B>This is bold text.</B>

would produce

This is bold text.

Similarly,

<I>This text is in italics.</I>

would produce

This text is in italics.

An HTML document is in two parts called the HEAD and the BODY.  What is in the HEAD is not normally displayed, although some browsers will display a title if it is included in the HEAD.  Level 2 HTML requires users to include a title of up to 64 characters.  This is because some search programs enter it in a database so that the search engine can find it if it contains what the searcher wants.  Thus it is a good idea to include some keywords in the title.  The heading tags <H1>…</H1> to <H6>…</H6> are used to create headings.  The layout is decided by the browser, so blank lines, tabs and extra spaces are ignored.  If you want these, you must use tags to do it.  This is because the browser has to fit the output to the display screen attached to the receiver.  These may be set up in many different ways.  Fig. 3.10.f.1 shows a simple example of HTML.  In this piece of HTML the blank <HR> tags are used to insert blank lines because the Web browser ignores the carriage return and new line characters.

<HTML>

<TITLE> An Example of HTML </TITLE>

<HEAD/>

<BODY>

<HR>

<H1>An Example of HTML </H1>

<HR>

This piece of text has been produced using HTML.  The text may be

<B>bold</B> or <I>italic</I>.

Although this piece of text is on a new line here, it may not be when displayed by the browser.  Remember, the Web browser decides the layout unless tags are used.

</BODY>

Fig. 3.10.f.1

The result of a browser running this HTML will vary, but will be something like that shown in Fig. 3.10.f.2.

# An Example of HTML

This piece of text has been produced using HTML.  The text may be bold or italic. Although this piece of text is on a new line here, it may not be when displayed by the browser.  Remember, the Web browser decides the layout unless tags are used.

Fig. 3.10.f.2

A line space and a thick line precede headings.  A line space and a thick line also follow them.

Exactly how the information is displayed will depend on the browser.  Also, some browsers do not recognise all tags.  If a browser encounters an unknown tag, it should ignore it.  However, there is no guarantee of this.  The result is that a page that looks outstanding when you design it, may not look very good on a different browser.

In Fig. 3.10.e.2 you will see many links, most of which link you to different sites around the world.  For example, Mondex links you to mondex.com, the home page for Mondex who specialise in applications of smart cards.

To use links as shown in the previous Section, you need to use the anchor tag <A>.

To create the link		Smart Cards

say, we need to use		<A>Smart Cards</A>

in the HTML document.  However, this will not create the link; it only creates the hypertext.  This hypertext must now be linked to the site.  You do this by giving the anchor attributes, using a hypertext reference (HREF).  This points to where the document to be displayed is kept.  A typical example is shown in Fig. 6.4.6.3.

Note this only shows the HTML necessary to create the link.

You will find more information on Smart Cards at

<A HREF =

"http://www.mondex.com/">

Smart Cards</A>

if you are interested.

Fig. 3.10.f.3

A shortened version can be used if the link is to a document in the same directory as the one being viewed.  In this case we need only write

<A HREF = "documentname.html"> … </A>

If the document is in a subdirectory of the directory containing the page being viewed, we can write

<A HREF = "subdirectoryname/documentname.html> … </A>

Links can also be created to points in the same document by using the NAME attribute.

Images can be placed on Web pages to

- improve the look of the page,

- add to text

- provide a hyperlink.

Inserting an image for interest is done by means of the <IMG> tag which has no end tag.  You must specify where the image is stored known as the source (SRC).  For example

<IMG SRC = "mypicture.gif">

where the BASE has been set by using, say,

<BASE HREF = "http://www.mysite.org.uk/pictures/">

or you can write it out in full as

<IMG SRC = " http://www.mysite.org.uk/pictures/mypicture.gif">

If you want the image to be a hypertext link, then use, for example,

<A HREF="hrefURL"> <IMG SRC="imageURL"> </A>

3.10.g Electronic Mail (email)

Electronic mail is a fast and cheap method of corresponding with others.  It does not matter what time you send it, you do not have to consider that at 08:00 in London it is only 03:00 in New York.  Also, email can be delivered when nobody is available to receive it.  The facilities offered by email are numerous as are their advantages.

Electronic mail systems allow the user to compose mail and to attach documents, in many formats, to the message.  Suppose several people are working on different chapters of a book.  It is easy for them to pass their work to one another as an attachment so that others can make comments and revisions before retuning them.  This book was created in this way.  The ability to attach all kinds of documents can prove very useful.  The author of this Chapter uses email to collect homework.  Students can word process their work and send it as an attachment.  I can then mark it and return my comments.  Even better, students attach programs they have been asked to write and I can run them to see if they work!

Often emails are sent to people who need to pass the message on to someone else.  This is easy as there is a forward facility with all email services.  All the user has to do when an email is to be passed on to someone else is to click a button, enter the email address and press the Send button.

It is easy to reply to an email as you only have to click a Reply button and the original sender's address automatically becomes the address to which the reply is to be sent.

Another useful facility that can be used is the facility to send the same email (and attachments) to a group of people.  For example, if I wish to send a message to the whole of one of my classes I can do this.  All that is necessary is for me to create a group by inserting in it the email addresses of all the students in the class.  I can then type the message once and send it to the whole group by means of a single click on Send.

Users of email can also set message priorities and request confirmation of receipt.

It is also possible to use voice mail in a similar way to email.  In this case the spoken message is digitised and stored electronically on a disk.  When the recipient checks for mail, the digitised form is turned back into sound and the receiver can hear the message.  These messages can also be forwarded, stored and replied to.

3.10.h Network User Interfaces

Many different types of user will use a network.  A company network will be used by design staff, payroll clerks, health and safety staff, sales staff, invoice clerks and many others.  All these different staff need a different interface.  Most staff will require access to standard packages such as a word processor.  However, not all staff need access to personnel records.  This means that the interface should not allow them any way into these records.  So all staff will need a different interface according to their needs.

This is common in schools and colleges.  Usually teaching staff will have a different interface to that used by the students.  Also, network administrators will need a different interface.

The differences are not just what is available to the user.  Some users will only want to use a simple graphical user interface (GUI), others will prefer to use many menus.  Technical staff may want to be able to access the computer at a very low level and would then need to use a command based interface.

You should read Section 1.2.c which describes the different types of interface.

The problem lies in the fact that all these different interfaces must be made available to different users on the same network.  This is done by the system recognising which user has logged on and then presenting the user with the appropriate interface.  Which interface is provided is laid down by the network manager who sets up rights for each user.

3.10.i Confidentiality of Data

Once an organisation opens some of its network facilities up, there is a problem of confidentiality of data.  An organisation may well wish that potential customers have access to their product database.  However, they will not want them to have access to employee files.

A first step is to encrypt the confidential data and this is addressed in the next Section.

Another solution is to install firewalls.  These sit between WANs and LANs.  The firewall uses names, Internet Protocol addresses, applications, and so on that are in the incoming message to authenticate the attempt to connect to the LAN.  There are two methods of doing this.  These are proxies and stateful inspection.  Proxies stop the packets of data at the firewall and inspect them before they pass to the other side.  Once the packets have been checked and found to be satisfactory, they are passed to the other side.  The message does not pass through the firewall but is passed to the proxy.  This method tends to degrade network performance but offers better security than stateful inspection.

Stateful inspection tracks each packet and identifies it.  To do this, the method uses tables to identify all packets that should not pass through the firewall.  This is not as secure as the proxy method because some data do pass through the firewall.  However, the method uses less network resources.

Another way of ensuring privacy of data is to use authorisation and authentication techniques.  These are explained in the next Section.

3.10.j Encryption, Authorisation and Authentication

Encryption is applying a mathematical function, using a key value, to a message so that it is scrambled in some way.  There are many techniques for this.  The problem is to make it virtually impossible for someone to unscramble the message.  Clearly, whatever function is applied to the original message must be reversible.  The problem is to make it very difficult for anyone to find the inverse of the original function.  It also means that there is a problem of many people needing to decrypt a message.  All these people need the key to unlocking the message.  This makes it highly likely that an unauthorised person will get hold of this key.  One method of overcoming this is to use Public Private Key technology.  This involves the sender having a public key to encrypt the message and only the receiver having the private key to decrypt the message.

Authentication is used so that both parties to the message can be certain that the other party is who they say they are.  This can be done by using digital signatures and digital certificates. Digital signatures require encryption.  Basically, a digital signature is code that is attached to a message.

In order to understand how public key cryptography works, suppose Alice and Bob wish to send secure mail to each other:

- First, both Bob and Alice need to create their public/private key pairs. This is usually done with the help of a Certification Authority (CA).

- Alice and Bob then exchange their public keys. This is done by exchanging certificates.

- Bob can then use his private key to digitally sign messages, and Alice can check his signature using his public key.

- Bob can use Alice's public key to encrypt messages, so that only she can decrypt them.

A primary advantage of public-key cryptography is the application of digital signatures, which help combat repudiation, i.e. denial of involvement in a transaction. Since the owner keeps their private key secret, anything signed using that key can only have been signed by the owner.

The predominant public-key algorithm is RSA, which was developed in 1977 by, and named after, Ron Rivest, Adi Shamir, and Leonard Adleman.  The RSA algorithm is included as part of Web browsers from Netscape and Microsoft and also forms the basis for many other products.

3.10.k Distribution of a Network

There are two different types of distributed network.  The first is created by means of partitioning.  In this method the database is split up and the resulting partitions are stored where the data are most needed.  The partitions are held in different locations as shown in Fig. 3.10.k.1.  This method gives faster access to the data as only when rarely used data is required by a user will access to the central database be needed.  The problem is keeping the central database up to date.  This is usually done by batch processing.  This means that each local partition manager must take responsibility for seeing that the central database is updated.  It also means that the central database is not always up to date.

Central database

Host Computer

Remote Computers

Remote database Partition A

Remote database Partition B

Remote database Partition C

Remote database Partition D

Fig. 3.10.k.1

An alternative configuration is shown in Fig. 3.10.k.2 where the Central database is duplicated.  This method also means that the central database must be updated during off peak hours.

Central database

Host Computer

Remote Computers

Remote database duplicate

Remote database duplicate

Remote database duplicate

Remote database duplicate

Fig. 3.10.k.2